Practical Saturation of Freeform Microlens Arrays on Extended OLED Emitters and Design Routes beyond Lens Shape

Authors — [TBD]   |   Affiliation — [TBD]

# Abstract

Freeform microlens arrays (MLAs) are widely expected to control both the light-extraction efficiency and the angular emission of organic light-emitting diodes (OLEDs). We test this expectation for MLAs tiled over an extended emitter. We couple an experimentally validated dipole-microcavity source model to three-dimensional ray tracing and compare an axisymmetric freeform MLA with an equally optimized hemispherical MLA under identical constraints. Total external quantum efficiency (EQE) and four polar bands are optimized as separate objectives. The freeform optimum exceeds the hemisphere by at most 7% in any band and by 1.3% in total EQE. A control that restarts every search at the hemispherical optimum shows that these small margins come from the design space, not from a weak search. All sampled designs collapse onto one line relating band power to total EQE. Among efficient designs the angular composition stays within a narrow window. This holds for convex, concave, and randomly assembled arrays alike. We trace the saturation to three causes: the output/source area ratio of a coextensive array is fixed, the thick substrate mixes light laterally, and a passive film cannot raise radiance. The platform uses a lossy Al cathode, the regime in which lens shape has the most leverage, so the saturation is expected to be stronger in low-loss devices. We then map the design routes that remain productive, led by a lower round-trip loss.

# 1. Introduction

A planar OLED loses much of its generated light. Waveguided modes in the organic layers, losses at the metal electrode, and total internal reflection (TIR) at the substrate/air interface all trap photons [1,2]. External MLAs extract the substrate modes without touching the electrical stack, which makes them attractive for large-area and flexible OLEDs [3–5]. Hemispherical and near-hemispherical MLAs already give high efficiencies [4,5]. With external outcoupling structures, however, OLED EQEs have generally plateaued near 55–60%. This ceiling has recently been traced to the loss that light returned by the outcoupling structure suffers on each round trip through the device [6]. High total extraction has also been reached without lenses, by combining an external scattering layer with horizontally oriented emitters [7].

More complex lens shapes are still expected to do better. Freeform illumination optics and inverse design are powerful tools for producing a prescribed intensity distribution. They have recently been applied to micro-LED and OLED packaging [8,9]. Tuning lenslet curvature, height, asymmetry, and the microcavity together should, in principle, give finer control over both total extraction and emission into chosen directions.

That expectation comes from optics built around point-like sources, and it may not carry over to an OLED. Freeform design methods were developed for sources of near-zero étendue. A thin-film OLED is an extended source, and its light reaches the array only after travelling inside a millimetre-thick substrate, often over several round trips. Whether shape freedom pays off in this regime is therefore an open question, and the answer has a practical cost. Freeform molding raises fabrication and metrology effort. If it buys little over a hemisphere, the next design step should be a change of loss, source, or aperture rather than a more complex surface.

The round-trip-loss analysis of ref. [6] already showed that, in a low-loss device, the EQE becomes insensitive to the aspect ratio of a hemispherical microlens. That analysis concerned total efficiency and a one-parameter lens family. It did not ask whether a freely shaped profile can steer power into chosen polar bands, nor whether the angular composition of the output can be moved at all.

We therefore ask: how much is lens-shape freedom worth for a tiled refractive MLA on an extended OLED? We answer with a controlled numerical benchmark. We use one source model that has been validated against fabricated devices [10,11] and one set of manufacturable constraints. Under these we compare a hemispherical MLA and an axisymmetric freeform MLA, each optimized for total EQE and for four polar bands. A restart control separates the limits of the search from the limits of the physics. Two further external-film families, inverted (concave) and randomly assembled arrays, test whether the outcome depends on how the array is built. The result is a reproducible numerical observation for this platform, not an impossibility theorem: a practical saturation of the explored design space. We explain why it occurs and map which levers remain productive beyond it.

# 2. Methods

## 2.1 Source model and experimental validation

Dipole emission in the OLED microcavity is computed with the Chance–Prock–Silbey (CPS) formalism. The result is the substrate-side angular–spectral intensity Isub(θ,λ), which serves as the source for three-dimensional ray tracing in LightTools, driven from MATLAB through the COM interface (Fig. 1). The stack is Al (100 nm) / B3PyMPM ETL / TCTA:B3PyMPM EML (25 nm) / TCTA (10 nm) / TAPC HTL / ITO (150 nm) / glass. The organic layers are birefringent, with measured ordinary and extraordinary indices, and Al and ITO use dispersive optical constants. The emitter is Ir(dmppy-ph)2tmd, with a horizontal dipole ratio of 0.865, a radiative efficiency of 0.98, and the emission zone at the centre of the EML; the emission spectrum spans 453–753 nm. With an Al cathode and an ETL of index about 1.8, light returned to the device loses about 13–16% per round trip [6], so the platform lies in the loss-limited regime where external structures reach EQEs near 55–60%. Section 4.3 discusses what this implies for the result. The emitting region is a disc of radius 1 mm, and the glass substrate is dsub = 1.295 mm thick with n = 1.51. All CPS inputs are listed in Supplement 1, Note S1 (Table S1).

The CPS output is handed to the ray tracer one wavelength at a time. For each sampled wavelength, a 1 mm-radius disc source launches rays into the substrate with the polar distribution Isub(θ) = (Is + Ip)/2, imposed by a 90-bin direction-grid apodizer and uniform in azimuth. The source power is set to ∈t Isubsinθ dθ. Light that the film returns to the device is reflected by a user coating on the device face, which carries the angle- and wavelength-resolved device reflectance (Rs+Rp)/2 computed by transfer matrices for the same stack. The total EQE is the sum over wavelengths of the traced extraction ratio multiplied by the CPS substrate-delivery efficiency at that wavelength. Polarization is therefore averaged both in the source and in the returned light. This is justified because rays in the substrate are reflected and refracted many times at the curved lens surfaces, which scrambles polarization, and because every class comparison uses the same treatment.

The same pipeline has been validated against fabricated devices with almost the same geometry. In ref. [11], OLEDs with a 1 mm emitting-aperture radius on a 1.4 mm substrate with nsub = 1.51 were made in three configurations: bare, MLA-attached, and with a structured outcoupler. Measured angular profiles and current efficiencies agreed with the simulations in all three cases, with measured maximum EQEs of 35.6%, 35.4%, and 48.0% (Fig. 3 and Table 2 of ref. [11]). In ref. [10], the pipeline reproduced the emission pattern and near-Lambertian angular electroluminescence of an inverted-MLA device. That device reached 58.0% EQE, against 30.5% for its planar control. The validation therefore covers the source model, the ray-optical treatment of microtextured exit faces, and both attached and inverted lenses, on the same emitter platform and substrate geometry used here.

## 2.2 Lens classes and constraints

The exit face carries a hexagonal array of lenslets of about 10 μm radius over a 25 × 25 mm patch. The emitting disc is therefore about 100 lenslet radii across, and the array extends well beyond it. The lens material is N-BK7 (n = 1.517 at 589 nm), nearly index-matched to the substrate (Δ n = 0.007, which changes the interface reflectance by about 10-5). The film/air boundary therefore sets the escape condition. The lens index is held fixed throughout. It is a property of the platform here, not a design variable.

Two lens classes are compared (Fig. 1b):

•	Hemispherical reference. The profile is the same spline with its five control points fixed on the quarter circle. It reproduces a true circle to within 0.007 lens radii for r < 0.95 and 0.055 lens radii at the rim. The lens height and the ETL and HTL thicknesses are free.

•	Axisymmetric freeform. The profile is a spline with endpoints fixed at (0,1) and (1,0) and five free control points, whose radial coordinates are constrained monotonic. Together with the two cavity thicknesses and the lens height, this gives 13 design variables.

Both classes share the same material, pitch, fill factor, height range, and cavity bounds (Table 1). Because the hemispherical reference is the member of the spline family whose control points lie on the quarter circle, it is an exact point of the freeform design space. The freeform constraints are never more permissive than the hemisphere's, so any difference measures shape freedom alone. Cavity thicknesses are optimized together with the lens in every campaign, so each class is compared at its own best cavity.

Table 1 | Design variables, bounds, and feasibility rules.

| Quantity | Freeform | Hemispherical reference |
| --- | --- | --- |
| Radial control points x2–x6 (lens radii) | 0–1, sorted ascending | fixed on the quarter circle |
| Height control points y2–y6 (lens radii) | 0–1.5 | fixed on the quarter circle |
| End points | (0, 1) and (1, 0), fixed | same |
| Height scale (stretchZ) | 0.1–3 | 0.1–3 |
| ETL and HTL thickness | 10–150 nm each | 10–150 nm each |
| Feasibility rules | no self-intersection or collinear triples; control-point spacing 0.05–1.0; turning angle between adjacent segments ≤ 120° | always satisfied |
| Free variables | 13 | 3 |

Candidates that violate a feasibility rule are not traced. They are returned to the optimizer as infeasible, so that surrogateopt learns the feasible region.

## 2.3 Figures of merit

The total EQE, EQEtotal, counts all photons emitted into air over the full hemisphere (0–90°). The far field is divided into four polar bands over the full azimuth: 0–20°, 20–40°, 40–60°, and 60–80°. The band EQE, EQEj, is the fraction of generated photons emitted into band j. The band selectivity is

Sj=(EQEj)/(EQEtotal) .

The four selectivities therefore sum to slightly less than one. The 80–90° remainder is 3.1% of the output for the median design (2.1–8.2% over all designs) and falls as efficiency rises; normalizing the bands to 0–80° instead changes none of the conclusions (Note S4). As an analytic reference we use the Lambertian partition SjLam = sin2θhi - sin2θlo, which gives 0.117, 0.296, 0.337, and 0.220. The value of shape freedom for objective j is the relative gain

Gj=(max[EQEj|freeform])/(max[EQEj|hemisphere]) .

Each maximum is the mean of three high-precision re-evaluations of the best design. Because the hemispherical reference is a point of the freeform design space, Gj ≥ 1 holds for an exhaustive search.

## 2.4 Optimization and the restart control

Each objective is optimized by surrogate-based global search (MATLAB surrogateopt), followed by patternsearch polishing from the surrogate winner. The campaigns are:

•	one optimization per objective, for total EQE and each of the four bands;

•	a dedicated total-EQE campaign with three independent starts;

•	a weighted-sum sweep, Jw = w ηext + (1-w) P40–60 with w ∈ 0, 0.25, 0.5, 0.75, 1, together with 150 random feasible designs that populate the design space without optimizer bias.

Searches use 10,000 rays and 31 wavelengths. Every reported optimum is re-evaluated three times with 50,000 rays and 151 wavelengths; the standard deviation of a re-evaluated EQE is about 10-4. Budgets, trace counts, and failure rates for every campaign are listed in Supplement 1, Table S2. An evaluation counts as usable when the trace returns a positive total EQE; no other filtering is applied. Of the 691 logged evaluations of the weighted-sum campaign, 606 are usable (114 of the 150 random designs). Trace failures occurred in 10.5% of the freeform per-band evaluations and in none of the hemisphere evaluations.

A search that misses part of its own design space could report a false absence of gain. To exclude this, we ran a restart control. For each objective, the search was restarted from that objective's hemispherical optimum, using the identical objective, constraints, and fidelities. The hemisphere point and eight perturbations within 8% of each variable's range were placed in the seed set. A pattern search was also launched directly from the hemisphere point. The hemisphere baseline was re-measured in the same session. The test statistic is the absolute difference of the two means divided by its pooled standard error, t = Δ/(sp2/n) with n = 3 repeats per design. An objective counts as improved when t exceeds the one-sided 95% threshold, 2.13 at four degrees of freedom. The three large effects reported below remain significant under a Holm correction for the five objectives (step-wise thresholds 2.78–3.75). The one borderline case was re-measured with five repeats per design (threshold 1.86 at eight degrees of freedom). The control is biased toward reporting a gain, because the winner is the maximum over many noisy evaluations. A null result under it is therefore conservative (Supplement 1, Note S2).

The sampling uncertainty of the selectivity–efficiency correlations was checked on 20 designs spread across the efficiency range. These were re-evaluated with 200,000 rays (three repeats) and over a 450–750 nm broadband spectrum (Note S5). The dependence of absolute EQE on patch size was measured separately (Note S3).

## 2.5 Additional external-film families

Two further manufacturable families test whether the outcome depends on array construction:

•	Inverted (concave) MLA. The concave counterpart of the same profile class, as realized by dimples in an ultrathin substrate [10]. It uses the same per-band protocol as the convex benchmark: four single-band optimizations of 60 surrogate evaluations plus 15 polish steps each.

•	Randomly assembled MLA. A 6 × 6-lenslet pseudo-random supercell, tiled over the patch. Lenslet positions are jittered off the hexagonal lattice, and each lenslet carries an independently drawn profile from the same class. Six assembly statistics replace the lens shape as design variables: fill factor, radius jitter, position jitter, mean aspect ratio, aspect jitter, and a hemisphere-to-random profile blend. The campaign samples 50 random realizations and then optimizes these statistics. Because the disorder correlation length is one pitch, a sixth of the supercell, the supercell is statistically equivalent to a fully random array.

A supercell trace is expensive, so the random family uses reduced sampling (5,000 rays and 16 wavelengths during search). A calibration shows that this shifts band selectivity by at most 0.39 percentage points (pp), comparable to the Monte-Carlo spread (Note S6).

All three families are external outcoupling films: they act on light that has already entered the substrate. Internal arrays beneath the electrode, which address waveguided rather than substrate modes [5], change the emitting stack itself. They lie outside this comparison.

Fig. 1 | Platform and benchmark design. (a) OLED–substrate–MLA architecture: CPS microcavity dipole source (emitting radius 1 mm), glass substrate (dsub = 1.295 mm), hexagonal lenslet array (~10 μm lens radius, 25 × 25 mm). (b) The two lens classes under identical constraints, drawn in units of the lens radius: the hemispherical reference (3 free variables) and the axisymmetric freeform (13 variables). Open circles are the five free spline control points; the freeform shown is the 40–60° optimum. (c) The four polar bands and the band selectivity Sj, taken over the full azimuth. (d) Optimization workflow: CPS source, LightTools ray tracing through the MATLAB COM interface, surrogate-based global search, pattern-search polishing, and high-ray re-evaluation.

# 3. Results

## 3.1 Freeform versus an equally optimized hemisphere

Figure 2 compares the two lens classes. The band-optimized freeform profiles differ visibly from the hemisphere and from one another (Fig. 2a). These shape differences buy little performance. The relative gains are Gj = 1.003, 1.002, 1.070, and 1.036 for the four bands, and G = 1.013 for total EQE (Fig. 2b). In no objective does the freeform exceed the hemisphere by more than 7%, and in none does it fall below.

The total-EQE ceiling of the freeform class was found twice, by two different procedures. The dedicated campaign reaches 0.5539, with a 0.8% spread across three independent starts. The restart control reaches 0.5523 in the same session as the hemisphere baseline of 0.5468. The two estimates agree within 0.3%.

The restart control also shows how much of the remaining margin is real (Fig. 2c). Gains above the hemisphere are significant in three objectives: +4.70% at 40–60° (t = 115), +3.64% at 60–80° (t = 30), and +1.01% for total EQE (t = 69). In the two lower bands the residual gains are +0.32% at 0–20° (t = 1.7) and +0.20% at 20–40° (t = 2.1), both below the threshold. A five-repeat re-measurement settles the 20–40° case as a real but tiny residue of +0.29% (t = 4.1). The same procedure that resolves a +0.29% residue does not miss gains ten times larger. The small values at 0–40° therefore show that shape freedom is worth little there, not that the search is weak. The original, randomly seeded per-band campaign under-recovered the hemisphere in these two bands. The restart corrects this, and the restart values are the ones reported (Note S2).

Table 2 collects every entry behind these numbers. Two different quantities appear in it, and they should not be confused. Gj compares the best freeform design of any matched campaign with the archived hemisphere optimum. The restart gain compares, within one session, the restarted design with a re-measured hemisphere. For 40–60° the two give 1.070 and +4.70%, because the per-band campaign found a better freeform design (0.19802) than the restart did (0.19355).

Table 2 | Freeform versus hemisphere for each objective.

| Objective | Hemisphere (archived) | Best freeform | Source of freeform entry | Gj | Restart control: hemisphere → restarted (mean ± SD) | Gain, t |
| --- | --- | --- | --- | --- | --- | --- |
| 0–20° | 0.06682 | 0.06699 | restart control | 1.003 | 0.06677 ± 0.00017 → 0.06699 ± 0.00013 | +0.32%, 1.7 |
| 20–40° | 0.16591 | 0.16628 | restart control | 1.002 | 0.16595 ± 0.00017 → 0.16628 ± 0.00020 | +0.20%, 2.1 (five repeats: +0.29%, 4.1) |
| 40–60° | 0.18503 | 0.19802 | per-band campaign | 1.070 | 0.18486 ± 0.00008 → 0.19355 ± 0.00010 | +4.70%, 115 |
| 60–80° | 0.13589 | 0.14075 | restart control | 1.036 | 0.13581 ± 0.00014 → 0.14075 ± 0.00025 | +3.64%, 30 |
| Total EQE | 0.54679 | 0.5539 | dedicated total-EQE campaign | 1.013 | 0.54679 ± 0.00013 → 0.55231 ± 0.00004 | +1.01%, 69 |

The hemisphere is therefore not merely a convenient baseline. It is a practical near-optimum of this structural class. This does not mean the optimum is exactly hemispherical. The best profiles differ from one another, and the low-efficiency part of the design space is diverse. Near the top of the performance range, however, this diversity does not convert into meaningful extra efficiency or band power.

Fig. 2 | Freeform versus an equally optimized hemisphere. (a) Band-optimized freeform profiles (colored) against the hemispherical reference at its own optimized height (black), in units of the lens radius. (b) Best EQE reached by each class at each objective, labelled with the relative gain Gj. All entries come from campaigns at the same 25 × 25 mm patch. (c) Restart control: gain over the hemisphere when each search is restarted from the hemispherical optimum, labelled with t against three high-precision repeats. Green marks gains above the one-sided 95% threshold (t = 2.13). A five-repeat re-measurement resolves the 20–40° residue as +0.29% (t = 4.1; Note S2).

## 3.2 Total efficiency fixes the angular composition

If shape freedom offered an independent angular degree of freedom, optimizing for one band would trade against total EQE and trace out a Pareto front. The data show no such front. Across all 606 usable evaluations of the weighted-sum campaign, the 40–60° band EQE is almost proportional to total EQE (R2 = 0.968, Fig. 3a). This holds for random and optimized designs alike, across total EQE from 0.12 to 0.56: the 114 random designs alone give R2 = 0.967, so the collapse is not produced by correlated points along optimizer trajectories. Because EQE40–60 is part of EQEtotal, the substantive content of the collapse is that S40–60 varies little; over all designs it stays within 0.336–0.375 (5–95%). The other bands collapse as well, with R2 = 0.85, 0.93, and 0.77 for 0–20°, 20–40°, and 60–80° (slopes and intervals in Note S4). Every weight w returns a design in the same high-efficiency cluster. For this design space, maximizing the band and maximizing total extraction are the same goal.

The angular composition of efficient designs is correspondingly narrow. We take the median selectivity of the twenty designs with the highest total EQE as the natural composition: what a design acquires when only efficiency is pushed. It is 0.094 / 0.278 / 0.361 / 0.236 for the four bands. The 10–90% spread is a few thousandths in each band. These twenty designs were located by four different single-band objectives, so the agreement is not an artifact of a shared objective. Nor is it an artifact of the choice of twenty: the top-10, top-20, and top-50 medians and means differ by at most 0.003, and bootstrap 95% intervals are ±0.002–0.005 (Note S4). Compared with the Lambertian partition, the platform depletes the 0–20° band and enriches the 40–60° band.

Band-dedicated optimization does move the composition, and we measure by how much (Fig. 3b). Selectivity rises by +27%, +8%, +1%, and +20% relative to the natural composition, but total EQE falls by 3%, 2%, 1%, and 11%. The two effects largely cancel. Referred to the natural composition at the best total EQE, the net band-power gains are 1.22, 1.05, 1.00, and 1.07 (decomposition in Note S4, Fig. S3). Two points follow. First, the 40–60° band, the natural target of a directional film, gains nothing: the most efficient design already maximizes it. Second, the largest gain is at 0–20°, the band the platform under-serves. Shape freedom restores a depleted direction better than it creates a new one. Part of this displacement comes from the cavity rather than the lens, because the cavity thicknesses are optimized together with the lens. In the multi-pass model of Note S9, with one common cavity, the 60–80° lens raises S60–80 by only about 0.005 over the hemisphere, whereas the 60–80° cavity alone raises it by about 0.06. The lens-only share of the steering is therefore even smaller than Fig. 3b suggests; a fixed-cavity LightTools control is in progress.

The composition is not strictly constant, and we report the deviation. Over the full sampled population, the correlation R between total EQE and Sj is +0.60, +0.55, −0.12, and −0.57 for the four bands (Fig. 3c; per-band panels in Fig. S4). As efficiency rises, the distribution tilts toward lower polar angles. The tilt is robust to sampling: a stratified subset of 20 designs, re-evaluated at 20 times the ray count, gives +0.61, +0.67, +0.04, and −0.71, with the same outer-band signs and a near-zero 40–60° value (Note S5). It is also not a patch artifact, because band selectivity changes by at most 0.24 pp when the patch is enlarged from 25 to 100 mm (Note S3).

The tilt does not reorder the bands among efficient designs. In all 520 designs with total EQE ≥ 0.40, the 40–60° band carries the largest share. Among the 421 designs with EQE ≥ 0.50, 416 follow the order 40–60° > 20–40° > 60–80° > 0–20°. Reordering appears only in the inefficient part of the space. In 8 of the 606 designs, all with EQE ≤ 0.354, the 60–80° band is largest. This is why the 20–40° and 60–80° fits cross at low efficiency in Fig. 3c.

Passive optics can trade position for angle, so étendue conservation alone does not fix the angular distribution, and we claim no absolute invariance. The data support a narrower, practical statement. Within the explored manufacturable class, angular redistribution is limited to a weak tilt of a few percentage points in selectivity, and its net value for band power is at most about 20%.

Fig. 3 | Total efficiency fixes the angular composition. (a) Total EQE versus 40–60° band EQE for random feasible designs (grey) and optimizer-visited designs (green) of the weighted-sum campaign. All points collapse onto one near-linear locus (black line, R2 = 0.968, n = 606). (b) Selectivity of each band-dedicated optimum (red) against the natural composition of the twenty most efficient designs (grey, 10–90% range) and the Lambertian partition (dashed), labelled with the net band-power gain. (c) Selectivity versus total EQE for all four bands, with least-squares fits and the correlation R in the legend. Among designs with total EQE ≥ 0.40 the 40–60° band is always the largest; the 20–40° and 60–80° fits cross only in the inefficient region.

## 3.3 Generality across external-film families

We next ask whether the result depends on the array construction. We repeat the analysis for the inverted (concave) and the randomly assembled families (Fig. 4), using the same statistics, each computed from the family's own evaluation log. The concave family uses exactly the variable bounds and feasibility rules of Table 1. Three signatures of saturation recur in all three families:

•	Collapse. The (total EQE, 40–60° EQE) points again collapse onto a line. R2 is 0.94 for the convex per-band campaign, 0.94 for the concave family, and 0.84 for the random family (Fig. 4, left column). The logs differ in sampling density and, for the random family, in fidelity, so we read these values only as evidence that the collapse recurs, not as a ranking of the families.

•	Narrow composition. The concave family's best total EQE is 0.517, 6% below the convex 0.548, as expected for a recessed rather than protruding surface. Its natural composition is 0.113 / 0.303 / 0.340 / 0.215. The random family reaches 0.522 ± 0.001 over three disorder realizations, a coefficient of variation of 0.2%. Only the assembly statistics matter, not the particular realization. Its natural composition is 0.111 / 0.299 / 0.347 / 0.213. Band-dedicated optimization of the concave family gives a largest net band-power gain of 1.19, in the 60–80° band, close to the convex family's 1.22.

•	Same direction of tilt. Selectivity again rises with efficiency at 0–40° and falls at 60–80° (Fig. 4, right column). The 40–60° correlation is weak in the two periodic families and negative in the random one. In every family the 40–60° band remains the largest in at least 99% of sampled designs.

The families also bound how far the composition can be moved at all. Every high-efficiency composition we obtained lies within 0.09–0.11 (0–20°), 0.28–0.30 (20–40°), 0.34–0.36 (40–60°), and 0.21–0.24 (60–80°). This spans three constructions, four single-band objectives, and two averaging conventions (top-twenty median and population mean). The change from convex to concave lenslets moves the composition to the edge of this window. The shift is at most 0.02 in any band, about the same as switching the averaging convention within one family (Note S4, Table S6). Positional disorder and per-lenslet shape disorder leave the composition inside the window. The composition is therefore set by the platform—extended source, thick substrate, coextensive external film—not by any design variable within it.

The randomly assembled MLA should not be confused with a volumetric scattering layer. Each lenslet remains a refractive surface with deterministic local normals and a single index step. A scattering layer instead redirects light by multiple volumetric scattering. Its behaviour depends on the particle–matrix index contrast and loading, which are not parameters of this study. Our conclusions apply to refractive external films and make no comparison with scattering layers.

Fig. 4 | Generality across external-film MLA families. Rows: convex freeform (reference, per-band campaign), inverted (concave), and randomly assembled (pseudo-random supercell). Every statistic is computed from each family's own evaluation log. (a1–c1) Total EQE versus 40–60° band EQE with a least-squares line; the near-linear collapse recurs in all three families. (a2–c2) Natural composition (top-twenty median, colored) against the convex reference (grey) and the Lambertian partition (dashed outline). The open circle is the population mean of the same family; the spread between statistics is comparable to the spread between families. (a3–c3) Correlation R between total EQE and Sj, per band, against the convex reference.

# 4. Discussion

## 4.1 Why the design space saturates

Three considerations explain the saturation, and a fourth explains why the composition, and not only the efficiency, is fixed.

No area leverage. Angular compression requires an output aperture larger than the source. A small source under a large lens can narrow its emission cone because the lens supplies extra output area. In a tiled MLA on a large-area OLED, each lenslet's aperture and the source area it serves grow together. The output/source area ratio is pinned near unity, whatever the lenslet size, so enlarging the lenslets does not restore the point-source collimation gain. The same relation explains why an index-matched hemispherical macroextractor works well on a small emitter, and why it cannot be tiled over a large area.

Lateral mixing in the substrate. A ray at the critical angle moves sideways by dsubtanθc = 1.16 mm in one pass through the substrate, and by 2.32 mm per round trip. That is about 100 and 200 lenslet radii. Each lenslet therefore receives light launched from a substrate region two orders of magnitude wider than itself. The light reaching the lens surface has already been spatially averaged before the lens shape can act on it. The patch-size series measures this mixing directly (Note S3). One fixed design gives total EQE 0.517, 0.543, 0.551, and 0.564 at patch sizes of 15, 25, 35, and 100 mm. Total EQE keeps rising slowly up to 100 mm, so light travels laterally over many round trips before escaping. The 25 mm values are therefore lower bounds. The angular composition changes by at most 0.24 pp over the same range, so every ratio, correlation, and class comparison is unaffected. Ref. [11] shows the same loss channel experimentally: an MLA film only 2 mm in radius on a 1 mm emitter gave no EQE gain (35.4% against 35.6% bare), because light escaped past the film edge before it could be extracted. For the same reason, round-trip experiments on small emitters need rear reflectors much larger than the active area [6].

The radiance limit of a passive film. For a given source radiance and exit area, the power a passive external layer can deliver into a chosen solid angle is bounded [12]. We use this bound as a benchmark, not as an impossibility proof. Once a hemisphere operates near the bound, a more complex profile can only reshuffle small amounts of power; it cannot create radiance.

Depletion of favourable angles. A hemispherical MLA returns unextracted light largely at its incident angle [6]. Light at angles the lens transmits well therefore leaves in the first passes, and later passes see mostly light at angles it transmits poorly. We quantified this with the angle-resolved multi-pass model of ref. [6], using ray-traced BSDFs of the optimized lenses of this work (Note S9). For the hemispherical total-EQE design with the Al cathode, the escape probability falls from 0.43 in the first pass to 0.39, 0.35, and 0.31 in the next three. A lens shaped to favour one band only changes which angles are emptied first, so the gain from reshaping is bounded. To test this, we replaced the angle-preserving device reflection by one that resets the returned light to a Lambertian distribution at every pass. The escape probability then stays at 0.42. The total extraction rises by 2.7–5.5% (Al) and 1.2–3.0% (Ag) at a common cavity, but the differences between lenses shrink: the spread of extraction efficiency over the nine optimized lenses falls from 2.5% to 1%, and the spread of S40–60 from 0.029 to 0.011. Breaking the angular memory of the returned light therefore does not restore leverage to the lens shape; it removes most of what remains.

## 4.2 Design routes beyond lens shape

Saturation of the lens shape does not end light extraction. It changes which quantity is worth varying. Table 3 maps each target figure of merit to the lever that remains productive once the hemispherical benchmark has confirmed saturation.

Table 3 | Design-route map after refractive-MLA saturation.

| Target figure of merit | Productive lever | Evidence | Practical constraint |
| --- | --- | --- | --- |
| Total EQE from substrate modes | Hemispherical MLA at its optimum; freeform shape adds ≤ 1.3% | Sec. 3.1, Fig. 2 | Molding tolerance only |
| Total EQE beyond the hemispherical benchmark | Lower round-trip loss: low-loss rear reflector (Ag or dielectric), thin and weakly absorbing transparent electrode [6] | EQE of an MLA device raised from 51% to 77% by a low-loss auxiliary reflector alone [6] | Reflector outside the current path, or integrated into the stack |
| Light delivered to the substrate and its angular distribution | Source/cavity engineering: dipole orientation, cavity design, substrate index [7,13] | Cavity co-optimized in every campaign | Effect on EQE shrinks as the round-trip loss falls [6] |
| Power in a chosen polar band | Follows total EQE; band-dedicated shape gives a net gain ≤ 1.22 | Sec. 3.2, Fig. 3 | Selectivity is paid for in total EQE |
| Angular compression (collimation) | Aperture expansion: output/source area ratio > 1 (macroextractor, pixel-level optics) | Composition window fixed in coextensive films, Fig. 4 | Loses planar tiling |
| Selectivity beyond the single-pass limit | Angle-selective multi-pass extraction: exit-side angular filter plus low-loss rear reflector [6,14–16] | Ideal filter 62.1% vs 29.1% planar at 10% loss (52.2% vs 27.2% at 15%); DBR 48.2% → 32.5% over 100 nm (Note S7) | Round-trip loss, source bandwidth, and angle randomization of the returned light (Note S9) |

Reducing the round-trip loss is the main lever for total EQE once the lens shape has saturated. Light that the film does not extract returns to the device and is partly absorbed on every round trip. The substrate-to-air extraction efficiency is then ηsta = p/[p+(1-p)A'], where p is the escape probability per encounter with the film and A' the loss per round trip [6]. With an Al cathode, A' ≈ 0.13–0.16 caps the EQE near 60%. A low-loss auxiliary reflector raised the EQE of an MLA device from 51% to 77%, without any change to the OLED stack or the outcoupling structure [6]. In the angle-randomizing approximation, lens shape acts only through p, and p matters less as A' falls, so this lever and the saturation reported here point the same way.

Source and cavity engineering sets how much light reaches the substrate and at which angles, through cavity thickness, dipole orientation, substrate index, or resonant structures [7,13]. In a lossy device it also changes the EQE strongly. Its effect on the EQE shrinks, however, once the round-trip loss is small [6].

Aperture expansion is needed when collimation is the main requirement. Everything we varied within the coextensive geometry moved the natural composition by at most 0.02 in any band: shape freedom, convex versus concave lenslets, and positional and shape disorder. Angular compression therefore requires changing the output/source area ratio, which means leaving the coextensive geometry. Small emitters and pixel-level optics allow this, at the cost of planar scalability.

Angle-selective multi-pass extraction can provide the selectivity that a non-selective refractive film cannot. Here an exit-side filter reflects out-of-band light back toward the device for another attempt. The multi-pass framework and the central role of the round-trip loss are those of ref. [6]; we add only angular selectivity on the exit side. This filter differs from the low-loss auxiliary reflector of ref. [6], which sits behind the device and returns light without angular selection. The two are complementary: the filter selects the angle, and the low round-trip loss lets the rejected light survive enough passes to be redirected. The filter only helps if the rejected light changes angle before it returns: with an angle-preserving device reflection and an MLA that keeps the angle, rejected light comes back at the same angle and is rejected again (Note S9). In a single pass, non-selective layers deliver 33.7% of the escaping light into the 40–60° band. A Markov multi-pass model with an ideal angular filter, full angle randomization, and 10% round-trip loss raises the delivery into that band to 62.1% of generated power, against 29.1% for a planar non-selective reference; at the 15% loss of the Al platform the two values are 52.2% and 27.2%. A realizable eight-pair dielectric multilayer gives 48.2% for a monochromatic source but only 32.5% over a 100 nm bandwidth (Note S7, Fig. S7). Round-trip loss and source bandwidth are therefore the practical limits of this route. This is a reference calculation that shows why angular selection must be paired with a low round-trip loss, not a device proposal.

## 4.3 Scope and limitations

Round-trip-loss regime. The platform uses an Al cathode with an ETL of index about 1.8, so its absolute EQEs lie in the loss-limited regime identified in ref. [6]. A low-index ETL in front of Al reduces the round-trip loss by total internal reflection, so this ceiling is specific to the conventional stack. One might ask whether the saturation is simply this ceiling. It is not. The comparison is relative: both lens classes share the same stack, and the hemisphere sits at 0.547 against 0.554 for the freeform. The multi-pass picture also shows that the lossy regime favours shape freedom. In ηsta = p/[p+(1-p)A'], which assumes the angle of returned light is randomized on each pass, the lens shape acts only through p. Raising p by 10%, from 0.40 to 0.44, raises ηsta by 2.9% at A' = 0.15 (Al), but by only 1.0% at A' = 0.045 (Ag) and 0.4% at A' = 0.02. The Al platform is therefore the case in which lens shape has the most leverage. In a low-loss device the saturation should be stronger, not weaker. The absolute EQEs reported here should not be compared directly with those of low-loss devices.

The saturation statement covers three manufacturable external-film families at substrate-matched lens index. It does not cover internal arrays, scattering layers, or high-index films. A separate exploration with three-dimensional asymmetric lenslets, run under different constraints, is consistent with the result but is suggestive only. It used an Ag electrode, an anisotropic emitter cell, and 52 shape variables. Its asymmetric shapes moved the far-field centroid but did not reliably raise the absolute power in a target (θ,φ) window above the hemisphere. Because its stack and constraints differ, we report it only qualitatively (Note S8) and draw no quantitative conclusion from it. Directional emission is possible with other elements, such as resonant structures, metasurfaces, and asymmetric prisms [14,17]; the result here concerns refractive lenslets only. Absolute EQE values at the 25 mm patch are lower bounds (Section 4.1), but ratios and comparisons are not affected. Finally, the simulated surfaces are ideal: there is no form error, misalignment, or lens-to-lens variation. Fabrication imperfections can only reduce whatever advantage a freeform profile holds over the hemisphere, so the near-null result is conservative. The saturation is a robust property of the validated pipeline and constraint set. The smallest residues, such as +0.29% at 20–40° or +1% in total EQE, lie within plausible model-form uncertainty (optical constants, dispersion, polarization averaging) and should be read as design-space margins, not as predicted experimental differences.

# 5. Conclusion

We tested the practical value of lens-shape freedom for manufacturable, tiled refractive MLAs on an extended OLED. Against an equally optimized hemisphere, the freeform optimum gains at most 7% in any polar band and 1.3% in total EQE. A restart control confirms that this is a property of the design space and not of the search. All designs collapse onto a single line relating band power to total EQE. Among efficient designs the angular composition is fixed to within a few percentage points, whatever objective located them. Optimizing for one band raises its selectivity by up to 27%, but costs up to 11% of total EQE, leaving a net band-power gain of at most 1.22.

Convex, concave, and randomly assembled films all keep the composition within one narrow window. That window is therefore a property of the platform: an extended source, a thick substrate, and a coextensive external film. The finding is not a claim that the hemisphere is optimal in every optical system. It provides a decision criterion. Once the hemispherical benchmark confirms saturation, further improvement should come from a lower round-trip loss, from the source and cavity, from aperture expansion, or from angle-selective multi-pass extraction, not from more lens-shape freedom. Because the platform uses a lossy Al cathode, where lens shape has the most leverage, the saturation is expected to be stronger still in low-loss devices. The same reasoning applies to other extended thin-film emitters, such as perovskite and quantum-dot LEDs.

# Funding

[TBD — 과제/기관 정보 입력 필요]

# Acknowledgments

[TBD]

# Disclosures

The authors declare no conflicts of interest.

# Data availability

The result archives underlying all figures and tables are available from the corresponding author upon reasonable request. These comprise the evaluation logs and optima of every optimization campaign and the multi-pass-model outputs, listed in Supplement 1. The archives come with the scripts that generated them. make_figures.py and make_supp_figures_and_data.py reproduce every figure, together with an Excel workbook of its plotted data. Data for the separately constrained asymmetric exploration are also available on request. Running the CPS source model and LightTools project files requires a LightTools license.

# References

1.	Brütting, W.; Frischeisen, J.; Schmidt, T. D.; Scholz, B. J.; Mayr, C. Device Efficiency of Organic Light-Emitting Diodes: Progress by Improved Light Outcoupling. Phys. Status Solidi A 2013, 210, 44–65.

2.	Yablonovitch, E. Statistical Ray Optics. J. Opt. Soc. Am. 1982, 72, 899–907.

3.	Möller, S.; Forrest, S. R. Improved Light Out-Coupling in Organic Light Emitting Diodes Employing Ordered Microlens Arrays. J. Appl. Phys. 2002, 91, 3324–3327.

4.	Wrzesniewski, E.; et al. Enhancing Light Extraction in Top-Emitting Organic Light-Emitting Devices Using Molded Transparent Polymer Microlens Arrays. Small 2012, 8, 2647–2651. https://doi.org/10.1002/smll.201102662.

5.	Qu, Y.; Kim, J.; Coburn, C.; Forrest, S. R. Efficient, Nonintrusive Outcoupling in Organic Light Emitting Devices Using Embedded Microlens Arrays. ACS Photonics 2018, 5, 2453–2458. https://doi.org/10.1021/acsphotonics.8b00255.

6.	Kim, J.; Lee, S.; Yoo, S.; Gather, M. C. Minimizing Round-Trip Loss for Near-Unity Quantum Efficiency in Organic Light-Emitting Diodes. Preprint 2026.

7.	Song, J.; et al. Lensfree OLEDs with over 50% External Quantum Efficiency via External Scattering and Horizontally Oriented Emitters. Nat. Commun. 2018, 9, 3207. https://doi.org/10.1038/s41467-018-05671-x.

8.	Kim, S.; Shin, J. M.; Lee, J.; Park, C.; Lee, S.; Park, J.; Seo, D.; Park, S.; Park, C. Y.; Jang, M. S. Inverse Design of Organic Light-Emitting Diode Structure Based on Deep Neural Networks. Nanophotonics 2021, 10, 4533–4541. https://doi.org/10.1515/nanoph-2021-0434.

9.	Ni, Y.; Feng, D.; Ma, D. Design of Freeform Microlens Arrays with Prescribed Luminance Distributions for MicroLED Optical Packaging. Appl. Opt. 2025, 64, 7875–7884. https://opg.optica.org/ao/abstract.cfm?uri=ao-64-27-7875.

10.	Kim, J.; Kim, E.; Park, J.; Song, J.; Kim, S.; Moon, H.; Yoo, S. Toward Near-Foldable Surface Light Sources with Ultimate Efficiency: Ultrathin Substrates Embedded with Micron-Scale Inverted Lens Arrays. ACS Photonics 2023, 10, 1775–1782. https://doi.org/10.1021/acsphotonics.3c00017.

11.	Kim, M.; Kim, J.; Yoo, S. Near-Planar Light Outcoupling Structures with Finite Lateral Dimensions for Ultra-Efficient and Optical Crosstalk-Free OLED Displays. Nat. Commun. 2025, 16, 11606. https://doi.org/10.1038/s41467-025-66538-6.

12.	Winston, R.; Jiang, L.; Ricketts, M. Nonimaging Optics: A Tutorial. Adv. Opt. Photon. 2018, 10, 484–511.

13.	Xiang, C.; Koo, W.; So, F.; Sasabe, H.; Kido, J. A Systematic Study on Efficiency Enhancements in Phosphorescent Green, Red and Blue Microcavity Organic Light-Emitting Devices. Light: Sci. Appl. 2013, 2, e74. https://doi.org/10.1038/lsa.2013.30.

14.	Buhl, M.; et al. Resonance-Based Directional Light Emission from Organic Light-Emitting Diodes. Adv. Photonics Res. 2023, 4, 2200143. https://doi.org/10.1002/adpr.202200143.

15.	Liao, P.-H.; Lee, W.-K.; Lee, C.-C.; Huang, C.-W.; Wen, S.-W.; Chen, Y.-T.; Chen, C.-C.; Lin, W.-Y.; Kwak, B. L.; Visser, R. J.; Wu, C.-C. Using Angle-Selective Optical Film to Enhance the Light Extraction of a Thin-Film Encapsulated 3D Reflective Pixel for OLED Displays. Opt. Express 2022, 30, 46435–46449. https://doi.org/10.1364/OE.477797.

16.	Kim, H.-J.; et al. High Efficient OLED Displays Prepared with the Air-Gapped Bridges on Quantum Dot Patterns for Optical Recycling. Sci. Rep. 2017, 7, 43063. https://doi.org/10.1038/srep43063.

17.	Abdelkhalik, M. S.; Garcia-Santiago, X.; van Raaij, T.-J.; López, T.; Berghuis, A. M.; de Jong, L. M. A.; Gómez Rivas, J. Enhanced and Directional Electroluminescence from MicroLEDs Using Metallic or Dielectric Metasurfaces. Commun. Eng. 2025, 4, 63. https://doi.org/10.1038/s44172-025-00401-w.